CITY OF FLUSHING - RENTAL HOUSING INSPECTION CHECKLIST – FOR REFERENCE ONLY

SMOKE Detectors:

☐  Properly installed and operable

☐  Installed in correct locations: In sleeping units, outside of sleeping units, and on every floor

FURNACE:

☐  Gas shut off valve and on/off switch

☐  Vent secured and properly pitched

☐  Filters are replaced/cleaned regularly

☐  Operable, provides minimum 68 degrees

☐  Adequate surrounding clearance WATER HEATER:

☐  Gas shut off valve

☐  Proper pop-off valve and drop line

☐  Vent secured and properly pitched

☐  Operable with no rust holes or leaks

☐  Adequate surrounding clearance

ELECTRICAL PANEL:

☐  Service amps adequate for usage

☐  Properly grounded and in good condition

☐  No open circuits in box

☐  All circuits labeled and readable

☐  If fuses – proper sizes

☐  Adequate clearance and easy access

GENRAL ELECTRICAL:

☐  Covers on switches, junction boxes and outlets

☐  Splices to code in covered boxes

☐  Wire in conduit where required

☐  GFCIs where required and operable

☐  Switches, outlets, and fixtures operable

☐  Minimum 2 outlets in all rooms, properly spaced

☐  Adequate light fixtures, with globes as required

☐  Buffer space between lights and combustibles

☐  Safe, limited use of extension cords

☐  Exterior service lines secure and protected

GENERAL PLUMBING:

☐  Installed correctly with no leaks

☐  Proper venting

☐  Functioning drains with proper traps and covers SINKS, BATHTUBS & SHOWERS:

☐  Ventilation in bathroom

☐  Proper number and location

☐  Operable, with hot and cold water

☐  Functioning drains with proper traps

☐  No leaks or drips

☐  Garbage disposal operable if present TOILETS:

☐  Operating properly

☐  No overflow to sewer from tank

☐  No leaks or continuous running LAUNDRY:

☐  Proper dryer vent to exterior with cover

☐  Gas shut off valve, end of line capped if unused

☐  Proper and grounded electrical connection

☐  Washtub faucet backflow prevention present

WINDOWS:

☐  Proper location and sizes for light and ventilation

☐  Proper for egress and lockable

☐  Snug fit, openable and not broken or cracked

☐  Screens installed between June 1 and October 15

☐  Egress windows for basement bedrooms

☐  Bathrooms have operable window with screen (June 1-Oct. 15) or a vent fan

DOORS:

☐  Bedrooms and bathrooms require privacy doors

☐  Bedroom doors – no padlocks or inside keyed

☐  Exterior doors – correct for egress and lockable

☐  Exterior doors – snug fit, operable and unbroken

ROOMS:

☐  Walls, floors, and ceilings intact and maintained

☐  Unobstructed egress within and out of home

☐  Proper square footage, ceiling height and layout

☐  Fireplaces, wood stoves operable and maintained

☐  No rubbish or garbage accumulation, infestations

☐  Basement bedrooms comply with all safety codes

STAIRWAYS:

☐  Adequately lighted, no stored items blocking egress

☐  Steps stable, unbroken and adequate strength

☐  Handrails required if 5 or more steps

☐  Guardrails on open stairs over 30” high

BUILDING EXTERIOR:

☐  Roof waterproof and maintained; gutters work

☐  Walls, foundation, and chimney maintained

☐  Porches solid and maintained

☐  No openings for animal infestations

☐  Guardrails on porches or stairs over 30” high

☐  Handrails required if 5 or more steps

☐  House numbers present and readable from street

GARAGE:

☐  Proper electrical service, wiring, fixtures, etc.

☐  Structure solid and maintained YARD:

☐  Fences built to code and maintained

☐  Grass, shrubs, and trees maintained to code

☐  Accessory buildings maintained to code

☐  Pools maintained and secure from child entry

☐  Outdoor trash kept in solid covered container

☐  No junk, trash, or building materials stored outside

☐  No inoperable vehicles or stored vehicle parts/tires

☐  No improper vehicle, RV or trailer parking

☐  No unsafe or unsanitary conditions

☐  Pet waste must be removed regularly

GENERAL:

☐  Permits pulled for repairs or new work required

Note: It is REQUIRED that a fire extinguisher be provided for safety. City of Flushing rental inspections do not include inspections for mold, airborne contaminants or other possible contaminants.

Note: This is not a comprehensive list of all requirements. Questions? Please email the Rental Inspection Department at rentalinspections@flushingcity.com.